*

*       <<<<<<<<  SWMM 4.3 GRAPH BLOCK DATA FILE >>>>>>>>>

*

*   This file last modified 4/22/94 by WCH.

*

*       This is an input data file to the SWMM 4.3 (beta) Graph

*       Block for producing a line-printer graph of a flow or water

*       quality time series found on an interface file and/or from

*       measurements.  An alternative graphing procedure is to

*       use the Combine Block with ICOMB = 4 to produce an ASCII version

*       of an interface file that can then be imported into a spreadsheet

*       for high-quality graphics.  (The process can be automated using

*       macros and batch files.)  However, plotting of measured data

*       with predicted would have to be programmed by the user.  The Graph

*       Block presented herein permits plotting of predicted and

*       measured data on the same graph.

*

*       The line printer graphs should be printed in a compressed mode to

*       view all 120 columns.  Similarly, various editors or word processors

*       processors often permit more than 80 columns to be displayed

*       on the computer monitor.

*

*       All lines with an asterisk in column 1 are comment lines and are

*       ignored by the program.

*

*       Input data are free format and may be up to 230 columns wide.

*       You must have a value for every data column even if the program

*       will not actually use a given value.  There must be at least one

*       space or comma between every input value.  Alphanumeric data

*       should be enclosed in single quotes.

*

*       Caution!  Data lines that are "wrapped around" (continued on

*       two or more lines) should have a blank in column 1, unless a

*       card identifier is needed.

*

*       SWMM uses both U.S. customary and metric units.  The

*       examples use feet, cfs, acres, inches and inches/hour.  If metric

*       is specified substitute meters, cms, hectares, millimeters and

*       millimeters/hour.

*============================================================================

*	The SW card sets up the interface files to be used or created.

*       The Graph Block does not generate an output interface file, and

*       any JOUT value will be ignored.

*       Regarding JIN value, see NOTE for parameter NTAPE below.

*============================================================================

*  NBLOCK  JIN(1)  JOUT(1)

SW    1      9       0

*============================================================================

*   The MM card opens the scratch files to be used by different subroutines.

*   A certain number (0 for GRAPH) may be required for each block.

*============================================================================

*  NITCH  NSCRAT(1) NSCRAT(2) NSCRAT(3) NSCRAT(4) NSCRAT(5) NSCRAT(6) NSCRAT(7)

MM   1       1

*============================================================================

*  NOTE: This run requires interface file TRANS1.DNT.  This is supplied

*  with the .DOC files but may also be generated by running SWDAT1.DAT,

*  which must be run before GRAPH.DOC.  Sequence is thus:

*  Order     File         Interface File Generated

*  -----     -----        ------------------------

*    1       SWDAT1.DAT   TRANS1.DNT

*    2       GRAPH.DOC

*============================================================================

*     The @ command is used to permanently save an interface or

*     scratch file.  This line should be placed before the first SWMM

*     block call.  The format of the @ command is as follows:

*============================================================================

*Column 1    Unit number of the       Name of the interface

*            interface file saved     file (any valid DOS filename)

*            or utilized

*

@             9                       'TRANS1.DNT'

*============================================================================

$GRAPH     	Call the GRAPH Block with a '$' in first column.

*============================================================================

*	The 'A1' line defines the GRAPH Block program control.

*       The meaning of the values listed on each line are as follows:

*============================================================================

*	A1 Line       :

*          NTAPE      :  File (logical unit) where predicted graph

*                        information is stored.  Will usually equal

*                        JOUT value of previous block.  If zero,

*                        only measured data will be plotted.

*

*                        NOTE: Under most circumstances, NTAPE must =

*                        JIN value for this run of Graph Block in order

*                        for interface file to be opened properly.

*                        This is always true if Graph Block is run alone.

*

*          NPLOT      :  Number of locations (e.g. inlets) for which

*                        predicted hydrographs and pollutographs are

*                        to be plotted.  Maximum of 20 locations.

*          MEAS       :  Input and plot measured data.

*                        = 0, No measured data to be plotted.

*                        = 1, Read (and plot) line-input data.

*                        = 2, Read (and plot) data stored as

*                             line images on file MFILE

*============================================================================

*          Note: THE FOLLOWING TWO PARAMETERS ARE NOT USED IF MEAS = 0.

*============================================================================

*          MFILE      :  File (logical unit) where measured data

*                        are stored.  Not required if MEAS <_ 1.

*                        (If zero, defaults to input file reader).

*          MPLOT      :  Number of locations (e.g., inlets, manholes)

*                        for which measured data are to be input and

*                        plotted (MEAS = 1, 2).  Maximum = 20.

*          NQP        :  Number of pollutants plotted (max of 5).

*          METRIC     :  Metric units used for input/output

*                        = 0, U.S. customary units.

*                        = 1, Metric units used, indicated in

*                          brackets [] in the remainder of the table.

*          MCTH       :  Plot every MCTH point from the interface file.

*============================================================================

*   NTAPE  NPLOT  MEAS  MFILE  MPLOT  NQP  METRIC MCTH

A1    9      1      1      5      1     4    0      1

*============================================================================

*	The 'B1' line defines the GRAPH Block Pollutant Selection Data.

*                  IF NQP = 0 (A1), SKIP TO GROUP C1.

*                OTHERWISE, REPEAT GROUP B1 NQP TIMES.

*============================================================================

*	B1 Line       :

*          IPOL       :  Pollutant identifier from sequence on interface

*                        file. e.g., if IPOL(1) = 3, first pollutant

*                        plotted will be third on interface file.  User

*                        must know sequence, as determined from input

*                        to preceding block (e.g. group J3 of Runoff

*                        Block).  If IPOL = 0, pollutant is not found on

*                        interface file and is special input only.

*

*  Note:   If IPOL > 0, omit all the following parameters since they will

*          be obtained from the interface file.  For discussion of these

*          parameters, see group J3 of the Runoff Block.

*

*          NLOAD      :  Type of pollutant plot

*                        = 0, plot concentration vs. time,

*                        = 1, plot load (e.g., mass/time) vs. time.

*          PNAME      :  Pollutant name. 8 characters, max. (Format A8).

*          PUNIT      :  Pollutant units. 8 characters, max. (Format A8).

*          NDIM       :  Type of units

*                        = 0, mg/1

*                        = 1, 'other' per liter, e.g., MPN/1

*                        = 2, other concentration units, e.g. JTU, pH.

*============================================================================

*   IPOL  NLOAD   PNAME  PUNIT  NDIM

B1   1      0      ' '   '  '    0

B1   2      0      ' '   '  '    0

B1   4      0      ' '   '  '    0

B1   4      1      ' '   '  '    0

*============================================================================

*	The 'C1' line defines the locations (e.g. inlets, manholes)

*            for plotting predicted output.  Supply NPLOT values.

*                  IF NPLOT = 0 on line A1, SKIP TO GROUP C2.

*============================================================================

*	C1 Line       :

*         IPLOT(1)    :  First location to be plotted.

*            .                 .

*         IPLOT(NPLOT): Last location to be plotted.

*============================================================================

*   IPLOT(1)

C1     51

*============================================================================

*	The 'C2' line defines the locations (e.g. inlets, manholes)

*            for plotting measured output.  Supply MPLOT values.

*                  IF MPLOT = 0 on line A1, SKIP to group D1.

*============================================================================

*	C2 Line       :

*         KPLOT(1)    :  First location to be plotted.

*            .                 .

*         KPLOT(MPLOT): Last location to be plotted.

*============================================================================

*   KPLOT(1)

C2    51

*============================================================================

*    The 'D1' line supplies a plot title (enclose in single quotes).

*============================================================================

*	D1 Line       :

*          TITL       :  Title to be printed at bottom of each plot.

*============================================================================

D1 ' STEVENS AVENUE CATCHMENT - STORM OF NOV.28, 1973 '

*============================================================================

*      Note:  IF MPLOT = 0 on line A1 SKIP the remaining groups.

*             Otherwise read MPLOT groups of E1 (and possibly E2 and E3)

*             a total of NQP + 1 times.

*============================================================================

*	E1 Line       :

*          MDATA      :  Measured data for this graph and location

*                        corresponding to sequence for Group C2.

*                        =0, No measured data to be entered for this location.

*                        =1, Input measured data according to remaining

*                        parameters and format of Group E1.

*============================================================================

*  Note:  THE FOLLOWING PARAMETERS ARE NOT USED IF MDATA = 0 on line E1.

*============================================================================

*          LCARD      :  Number of graph ordinates per Group

*                        (MTIME > 0) or pairs of time-graph ordinates

*                        per Group (MTIME = 0).  Maximum = 16.

*          MTIME      :  Option for time of graph ordinates.

*                        = 0, Enter a time with each ordinate.  Cease input

*                        of time-ordinate pairs when entered time is > TQUIT.

*                        > 0, The time for each ordinate will be computed

*                        starting at TMZERO and using time increment

*                        DTMHR. Read a total of MTIME ordinates.

*          MUNIT      :  Units of time if MTIME = 0.

*                        Not required if MTIME > 0.

*                        =0, Time is in minutes.

*                        =1, Time is in hours.minutes

*                            (i.e. decimal point between hours and minutes).

*                        =2, Time is in decimal hours

*                            (and may have values > 24).

*          TMZERO     :  Initial time (decimal hours) of measured data

*                        if MTIME > 0.  Value of TMZERO is added to

*                        times entered if MTIME = 0.  May be used to

*                        provide a time offset for measured data,

*                        avoiding revision of their times.

*          TQUIT      :  A time greater than TQUIT ends entry of time-

*                        ordinate pairs if MTIME = 0.

*                        Not required if MTIME > 0.

*          DTMHR      :  Time increment (hours) if times of graph

*                        ordinates are calculated (MTIME > 0).

*                        Not required if MTIME = 0.

*============================================================================

*   MDATA  LCARD MTIME MUNIT TMZERO  TQUIT DTMHR

E1   1       5     0    1    -0.06    99.0   0

*============================================================================

*   Note: Date groups E2 AND E3 are NOT required if MDATA = 0 on line E1.

*============================================================================

*	E2 Line       :

*          FIRMAT     :  Format by which measured data of group

*                        E3 will be read.  Include beginning and

*                        final parentheses.  If blank (single quotes

*                        enclose a blank), the default

*                        format will be used: (2X,F8.0,7F10.0).

*                        If FIRMAT is 'FREE' then free format input

*                        will be used, i.e., values separated by commas

*                        or at least one blank.

*============================================================================

*      FIRMAT

E2 '(2X,10(F7.0))'

*============================================================================

*       Use data group E3 to input the measured flows and concentrations

*                if MEAS = 1 on line A1.

*

*    *** Note: If MEAS <_ 1 (Group A1) this Group will be read from

*                unit N5.  Otherwise, the formatted read will be

*                from unit number MFILE (Group A1). ***

*============================================================================

*	E3 Line       :

*          TIMX       :  Time (if MTIME = 0) and graph ordinate, LCARD

*       (optional)    :  pairs (if MTIME = 0) per Group, according to

*           and       :  format of Group E2. Entries are stopped when

*          YVAL       :  a time is > TQUIT (this time is not saved

*                        as a data entry).  If MTIME > 0, only YVAL will

*                        be read, a total of MTIME values, LCARD values

*                        per line according to format of Group E2.

*                        Units of hydrograph ordinate must be cfs

*                        [m3/sec if METRIC = 1], and pollutograph

*                        ordinates must be concentrations corresponding

*                        to NDIM of Group B1.

*============================================================================

*  Data group E3 input follows

15.145  6.513 15.160  5.933 15.175  4.337 15.190  5.187 15.205  6.085

15.220  6.309 15.235  7.514 15.250 12.656 15.265 15.802 15.280 18.384

15.295 21.361 15.310 25.563 15.325 25.905 15.340 24.290 15.355 23.487

15.370 20.636 15.385 31.605 15.400 33.554 15.415 35.328 15.430 38.886

15.445 41.100 15.460 32.849 15.475 27.390 15.490 24.283 15.505 19.126

15.520 15.957 15.535  8.656 15.550  8.498 15.565  9.021 15.580  8.609

15.595  8.256 16.010 10.604 16.025  9.555 16.040  8.397 16.055  7.984

16.070  7.981 16.085  5.264 16.100  5.181 16.115  5.003 16.130  4.267

16.145  5.277 16.160  5.655 16.175  5.300 16.190  5.527 16.205  5.726

16.220  6.423 16.235  6.399 16.250  6.743 16.265  6.541 16.280  7.187

16.295  5.462 16.310  4.570 16.325  4.361 16.340  5.534 16.355  5.009

16.370  4.769 16.385  5.442 16.400  5.627 16.415  4.323 16.430  5.465

16.445  5.625 16.460  5.287 16.475  5.093 16.490  5.618  99.

*============================================================================

*  Repeat groups E1, E2, and E3 for remaining MPLOT-1 locations for measured

*  hydrograph inputs. Then, input MPLOT groups of Groups E1, E2, and E3 for

*  first pollutograph, second pollutograph, etc., up to NQP pollutographs.

*  Note data for the MPLOT locations must appear in the order in which the

*  locations were entered on Group C2. There will be a total of

*  MPLOT * (NQP+1) entries of group(s) E1 (and possibly E2 and E3).

*============================================================================

*   MDATA  LCARD MTIME MUNIT TMZERO  TQUIT DTMHR

E1   1       5     0    1    -0.06    99.0   0

E2 '(3X,5(4X,F4.2,1X,F7.0))'

11281528 587.00011281538 310.00011281548 147.00011281558 147.00011281608 120.000

11281618 110.00011281628  83.00011281638  97.00011281648  83.000    99.0

*============================================================================

*   MDATA  LCARD MTIME MUNIT TMZERO  TQUIT DTMHR

E1   1       5    0     1    -0.06    99.0   0

E2 '(3X,5(4X,F4.2,1X,F7.0))'

11281528 1090.0011281538 750.00011281548 460.00011281558 500.00011281608 350.000

11281618 262.00011281628 192.00011281638 192.00011281648 200.060    99.0

*============================================================================

*   MDATA  LCARD MTIME MUNIT TMZERO  TQUIT DTMHR

E1    1      5     0    1     -0.06   99.0   0

E2   '(3X,5(4X,F4.2,1X,F7.0))'

11281528 1172.0011281538 798.00011281548 466.00011281558 456.00011281608 792.000

11281618 298.00011281628 252.00011281638 118.00011281648 114.000    99.0

*============================================================================

*               End your input data set with a $ENDPROGRAM.

$ENDPROGRAM