*

*       <<<<<<<<  SWMM 4.3 STATISTICS BLOCK DATA FILE >>>>>>>>>

*

*  This file last updated 4/22/94 by WCH.

*

*	This is an input data file to the SWMM 4.3 Statistics

*       Block for analyzing a rainfall, flow or water quality time series.

*       All lines with an asterisk in column 1 are comment lines and are

*       ignored by the program.

*

*       Input data are free format and may be up to 230 columns wide.

*       You must have a value for every data column even if the program

*       will not actually use a given value.  There must be at least one

*       space or comma between every input value.  Alphanumeric data

*       should be enclosed in single quotes.

*

*       Caution!  Data lines that are "wrapped around" (continued on

*       two or more lines) should have a blank in column 1, unless a

*       card identifier is needed.

*

*       SWMM uses both U.S. customary units and metric units.  The

*       examples use feet, cfs, acres, inches and inches/hour.  If metric

*       is specified substitute meters, cms, hectares, millimeters and

*       millimeters/hour.

*============================================================================

*	The SW card sets up the interface files to be used or created.

*	There is one output file (#9) which will contain the hourly

*       flows and pollutant loads for subsequent blocks.

*============================================================================

*  NBLOCK  JIN(1)  JOUT(1)

SW    1      8       9

*============================================================================

*   The MM card opens the scratch files to be used by different subroutines.

*   A certain number (3 for STATISTICS) may be required for each block.

*============================================================================

*  NITCH  NSCRAT(1) NSCRAT(2) NSCRAT(3) NSCRAT(4) NSCRAT(5) NSCRAT(6) NSCRAT(7)

MM   7       1         2          3        10         11        12       13

*============================================================================

*     The @ command is used to permanently save an interface or

*     scratch file.  This line should be placed before the first SWMM

*     block call.  The format of the @ command is as follows:

*============================================================================

*  NOTE: This file requires interface file RUNOFF6.DNT from Minneapois,

*  Lake Calhoun simulation.  This must be supplied or created by running

*  file SWDAT4.DAT.

*============================================================================

*Column 1    Unit number of the       Name of the interface

*            interface file saved     file (any valid DOS filename)

*            or utilized

*

@             8                       'RUNOFF6.DNT'

*============================================================================

$STATISTICS     	Call the STATISTICS Block with a '$' in first column.

*============================================================================

*	The 'A1' line defines the starting and ending time and date.

*       The meaning of the values listed on each line are as follows:

*============================================================================

*	A1 Line       :

*            ISTART   :  Starting date, 6 digits, yr/mo/day

*            TSTART   :  Starting time, decimal hours

*            IEND     :  Ending date, 6 digits, yr/mo/day

*            TEND     :  Ending time, decimal hours

*            INLOG    :  = 0, calculate only moments of untransformed data.

*                        = 1, also calculate moments of natural logs of storm

*                        event data.

*            JCUBE    :  = 0, storm event "volumes" are given in inches [mm].

*                        = 1, storm event volumes are listed in cubic feet

*                        [cubic meters].

*============================================================================

*   ISTART  TSTART  IEND   TEND  INLOG   JCUBE

A1  600101   0.0   601231   0.0      0       0

*============================================================================

*	The 'B1' line defines control information for the Statistics Block.

*============================================================================

*	B1 Line       :

*            MIT      :  Minimum interevent time, decimal hours.

*            BASE     :  Cutoff flow (baseflow), cfs [m3/s].  Flows less

*                        than BASE are not used.  This value is not

*                        subtracted from flow rate but only serves as a

*                        threshold for analysis.

*            EBASE    :  Cutoff storm event volume, inches [millimeters].

*                        Storm events below EBASE are not included in

*                        the statistical analysis.

*            LOCRQ    :  ID of flow location requested.  To perform

*                        statistical analysis on more than one location it

*                        is necessary to run the Statistics Block more

*                        than once.

*            LOCRN    :  ID of rainfall gage for analysis.

*                       Note: LOCRQ or LOCRN must be zero.  Both cannot

*                       be non-zero.

*            NPR      :  Number of pollutants requested (max = 5).

*            NPOINT   :  Number of events printed in tables.

*                        Print only the top NPOINT flows, events etc.

*                        IF NPOINT is 0 then all events are printed.

*            METRIC   :  Requests type of units for output

*                        = 0, U.S. customary,

*                        = 1, Metric.

*            LRET     :  Units of return period,

*                        = 0, Return period in years,

*                        = 1, Return period in months.

*            A        :  Plotting postion parameter (see text for

*                        explanation).  Briefly, the return period, T, is

*                        calculated as:

*                              T = (n+1-2A)/(m-A) , where

*                        n = total number of events, and

*                        m = rank.  E.g., use A = 0 for Weibull plotting

*                        position and A = 0.4 for Cunnane's recommended

*                        plotting position.

*============================================================================

*  MIT  BASE  EBASE LOCRQ  LOCRN NPR NPOINT METRIC LRET  PLOTTING POS(A)

*============================================================================

B1  6   0.0     0.0     1    0     5    50     0     1           0.4

*============================================================================

*	The 'B2' line defines print control information.

*============================================================================

*	B2 Line       :

*            KSEQ     :  Request to print sequential series

*                        of flow events?  No = 0, Yes = 1.

*            KTERM    :  Code for terminating program if number of

*                        events exceeds allowable memory space (5000 events).

*                        = 0, Do not terminate (perform analyses

*                        on those events already identified).

*                        = 1, Terminate program (no event analysis performed).

*            KTSEQS   :  Code for printing sequential series if the

*                        number of events exceed limit and KTERM = 1.

*                        = 0, Do not print sequential series.

*                        = 1, Print sequential series of those

*                          events already identified.

*============================================================================

*  KSEQ  KTERM  KTSEQS

B2  1      0       1

*============================================================================

*	The 'B3' line defines the interface file location

*                 of water quality information.

*                 Required only if NPR > 0 on Data Group B1.

*                 If NPR = 0 then skip to Data Group C1.

*

*     Max. of 5 pollutants requested.  Run Statistics Block again if

*     need to analyze more.

*============================================================================

*	B3 Line       :

*          IPOLRQ(1)  :  First pollutant requested, identified

*                        by position on interface file.

*                            .                     .

*          IPOLRQ(NPR):  Last pollutant requested, identified by              None

*                        position on interface file.

*============================================================================

*   IPOLRQ

B3     1  2  3  4  5

*============================================================================

*        Enter the C1 line only if LOCRQ on Data Group B1 is > 0.

*============================================================================

*        The C1 line describes the Statistics Block Options for Flow.

*

*    This data group controls the printing or plotting of information on

* magnitude, return period and frequency for each of the five flow parameters.

* In all cases, No = 0, Yes = 1.  The control information is entered as a four

* digit integer number with each column controlling a different table or graph.

*

*             First column  - table of magnitude, return period

*                             and frequency.

*             Second column - print graph of magnitude versus

*                             return period.

*             Third column  - print graph of magnitude versus

*                             frequency.

*             Fourth column - print moments.

*

*     For example, enter 1111 to print/plot all tables/graphs, 1000 to

*  print the table only, 1100 to print the table and the graph of

*  magnitude versus return period only, and 0000 to bypass printing/plotting

*  of the flow parameter.

*============================================================================

*	C1 Line       :

*         ISFLOW(1,1) :  Request for total flow?

*         ISFLOW(1,2) :  Request for average flow?

*         ISFLOW(1,3) :  Request for peak flow?

*         ISFLOW(1,4) :  Request for event duration?

*         ISFLOW(1,5) :  Request for interevent duration?

*============================================================================

*   ISFLOW(1,1) ISFLOW(1,2) ISFLOW(1,3) ISFLOW(1,4) ISFLOW(1,5)

C1         1111        1001        1001        1001        1001

*============================================================================

*           Data Group D1 is only required if NPR > 0 on Data Group B1.

*============================================================================

*      The D1 line describes the Statistics Block Options for Water Quality.

*

* If NPR > 0 use one D1 Data Group for each pollutant requested, up to

*    ten sets of D1 lines, in the order defined by Group B2.  The first

*    index of ISPOLL(K,I,J) identifies the pollutant.  Follow the

*    instructions for Data Group C1 in entering data for Data Group D1.

*============================================================================

*	D1 Line       :

*     ISPOLL(1,1,1)   :  Request for total load?

*     ISPOLL(1,1,2)   :  Request for average load?

*     ISPOLL(1,1,3)   :  Request for peak load?

*     ISPOLL(1,1,4)   :  Request for flow weighted average concentration?

*     ISPOLL(1,1,5)   :  Request for peak concentration?

*============================================================================

*   ISPOLL(1,1,1)  ISPOLL(1,1,2) ISPOLL(1,1,3) ISPOLL(1,1,4) ISPOLL(1,1,5)

D1           1111           1001          1001          1001          1001

D1           1001           1001          1001          1001          1001

D1           1001           1001          1001          1001          1001

D1           1001           1001          1001          1001          1001

D1           1001           1001          1001          1001          1001

*============================================================================

*        Enter the E1 line only if LOCRN on Data Group B1 is > 0.

*============================================================================

*   The E1 line describes the Statistics Block Options for Rainfall Analysis.

*

*    Follow the instructions for Data Group C1 in entering data

*                            for Data Group E1.

*============================================================================

*	E1 Line       :

*         ISFLOW(2,1) :  Request for total volume?

*         ISFLOW(2,2) :  Request for average intensity?

*         ISFLOW(2,3) :  Request for peak intensity?

*         ISFLOW(2,4) :  Request for event duration?

*         ISFLOW(2,5) :  Request for interevent duration?

*============================================================================

*   ISFLOW(2,1) ISFLOW(2,2) ISFLOW(2,3) ISFLOW(2,4) ISFLOW(2,5)

* E1         1001        1001        1001        1001        1001

*============================================================================

*               End your input data set with a $ENDPROGRAM.

$ENDPROGRAM