*

*       <<<<<<<<  SWMM 4.3 (beta) COMBINE BLOCK DATA FILE >>>>>>>>>

*

*  Note, this file last updated by WCH, 4/22/94.

*

*	This is an input data file to the SWMM 4.3 Combine Block

*       for collating, combining and providing readable versions of

*       SWMM interface files, including both hydrograph/pollutograph

*       time series, and precipitation time series from the Rain Block.

*       All lines with an asterisk in column 1 are comment lines and are

*       ignored by the program.

*

*       This file may be run as input to SWMM as long as interface files

*       RUNOFF.DNT and RUNOFFCB.DNT are present.

*

*       Input data are free format and may be up to 230 columns wide.

*       You must have a value for every data column even if the program

*       will not actually use a given value.  There must be at least one

*       space or comma between every input value.  Alphanumeric data

*       should be enclosed in single quotes.

*

*       Caution!  Data lines that are "wrapped around" (continued on

*       two or more lines) should have a blank in column 1, unless a

*       card identifier is needed.

*

*       SWMM uses both U.S. customary units and metric units.  The

*       examples use feet, cfs, acres, inches and inches/hour.  If metric

*       is specified substitute meters, cms, hectares, millimeters and

*       millimeters/hour.

*============================================================================

*	The SW card sets up the interface files to be used or created.

*	There is one output file (#9) from the first run of Combine,

*       no output from the second run of Combine (ICOMB=5), and a second

*       output file (# 10) from the third run of Combine that creates

*       an ASCII file (ICOMB=4).  Interface files are ordinarily written

*       in binary mode.  This special case (ICOMB=4) is the only instance

*       in which an interface file can be viewed and read using a DOS

*       editor.

*============================================================================

*  NBLOCK  JIN(1)  JOUT(1) JIN(2) JOUT(2)   etc.

SW    3      8       9       9      0       9      10

*============================================================================

*   The MM card opens the scratch files to be used by different subroutines.

*   A certain number (1 for COMBINE) may be required for each block.

*============================================================================

*  NITCH  NSCRAT(1) NSCRAT(2) NSCRAT(3) NSCRAT(4) NSCRAT(5) NSCRAT(6) NSCRAT(7)

MM   7       1         2          3        10         11        12       13

*============================================================================

*   In the example below, file RUNOFFCB.DNT is the same file as RUNOFF.DNT

*   and is simply a copy of RUNOFF.DNT.  Thus, the collated output file

*   will contain flows that are double those on the the input files.

*   After running this COMB.DOC file RUNCOMB.ASC is viewable with a

*   DOS editor.

*============================================================================

*     The @ command is used to permanently save an interface or

*     scratch file.  This line should be placed before the first SWMM

*     block call.  The format of the @ command is as follows:

*============================================================================

*Column 1    Unit number of the       Name of the interface

*            interface file saved     file (any valid DOS filename)

*            or utilized

*

@             8                       'RUNOFF.DNT'

@             1                       'RUNOFFCB.DNT'

@            10                       'RUNCOMB.ASC'

*  @          9                       'RUNCOMB.INT'

*============================================================================

*  Column 1

*  $ANUM      Alphanumeric labeling of node names is used.  Enter

*             all requests for node numbers with names in quotes.

*             This option must be used if Combine is operating on

*             interface files created by earlier SWMM blocks using

*             the $ANUM option.

*============================================================================

$COMBIN     	Call the COMBINE Block with a '$' in first column.

*============================================================================

*	The 'A1' line defines the Combine Block program control.

*       The meaning of the values listed on each line are as follows:

*============================================================================

*	A1 Line       :

*          ICOMB      : Program control.

*                       = 0, Collate option.  Merge two interface files (JIN

*                            and NSCRAT(1)), retaining all location IDs.

*                            For locations with identical IDs, sum flows

*                            and loads on new output interface file, JOUT.

*                       = 1, Combine option.  Sum all flows and loads on

*                            interface files JIN and NSCRAT(1) and combine

*                            on output interface file JOUT at location

*                            NODEOT given in data group B2.

*                       = 2, Extract (and optionally renumber)

*                            from a single file (JIN).

*                       = 3, Read the file headers of file JIN.

*                       = 4, Create a formatted ASCII (text) file from

*                            the unformatted interface file (JIN).  This

*                            file can be manipulated for plotting using

*                            a spreadsheet, for example.

*                            Caution.  The maximum product of number of

*                            locations x (1 + number of pollutants) must

*                            be <= 220.

*                       = 5, Calculate the simple statistics (totals) of flow

*                            and/or water quality on interface file JIN.

*                       = 6, Calculate the simple statistics (totals) of the

*                            precipitation interface file JIN.

*============================================================================

*  Collate interface files RUNOFF.DNT and RUNOFFCB.DNT.

*  ICOMB

A1   0

*

*   Note : End of input to the Combine Block if ICOMB = 3, 5 or 6.

*============================================================================

*	Create title lines for the simulation.  There are two title lines

*       for the Combine Block.  Titles are enclosed in single quotes.

*============================================================================

*	B1 Line       :

*          TITLE      : Title, 2 lines to be placed as first title on

*                       output file (maximum of 80 characters per line).

*============================================================================

B1 ' COMBINE BLOCK COLLATE EX.  INTERFACE FILE FROM RUNOFF.DOC'

B1 ' COLLATE WITH IDENTICAL INTERFACE FILE FOR THIS EXAMPLE'

*============================================================================

*	B2 Line       :

*          NODEOT     : Node number on output file for combined location.

*                       Enter zero if collating (ICOMB = 0).

*                       If $ANUM option is used, enter node name in quotes

*                       and use '' for zero.

*          NPOLL      : Number of quality constituents to be placed on

*                       new interface file.

*

*                       Note, option for ICOMB=4 and NPOLL > 0:

*

*                       Pollutants written on ASCII version of interface

*                       file have units of flow x concentration (e.g.,

*                       cfs x mg/l or cms x mg/l) as the default option.

*                       If NPOLL is input as a negative number, then

*                       concentrations will be placed on the ASCII file

*                       instead.  NPOLL will be reset to its proper

*                       positive value.

*============================================================================

*    NODEOT  NPOLL

B2     0       3

*============================================================================

*      Use the B3 to define the water quality interface file locations

*          if NPOLL > 0 on line B2.

*============================================================================

*	B3 Line       :

*         NPOS1(1)    :  Constituent 1 position on file 1.

*         NPOS2(1)    :  Constituent 1 position on file 2.

*               .                         .

*         NPOS1(NPOLL):  Constituent NPOLL position on file 1.

*         NPOS2(NPOLL):  Constituent NPOLL position on file 2.

*============================================================================

*    NPOS1(1) NPOS2(1) NPOS1(2) NPOS2(2)  etc.

B3     1        1         2       2       3       3

*       The C1 line defines the extraction/renumbering options.

*============================================================================

*	C1 Line       :

*          NUMX       : = 0, Use all locations.

*                       > 0, number of locations (nodes) to be

*                         extracted from one or two files.

*          NUMR       : = 0, do not renumber any locations.

*                       = NUMX, renumber according to data group C3.

*                       (If NUMX = 0, NUMR can equal total number of

*                        nodes.  See text for more information.)

*============================================================================

*    NUMX   NUMR

C1    0      0

*============================================================================

*       Define the locations (nodes) to be extracted if NUMX > 0

*                  on data group C1 using data line C2.

*============================================================================

*       Not required if NUMX = 0 in data group C1.

*

*	C2 Line       :

*        NODEX(1)     :  First node number.

*             .              .

*        NODEX(NUMX)  :  Last node number.

*============================================================================

*    COULD EXTRACT NODE NUMBER 3

*    NODEX(1)

*  C2     3

*============================================================================

*       Define the Numbers to be Assigned to Extracted Nodes

*                  if NUMR > 0 on data group C2.

*============================================================================

*     Not required if NUMR = 0 in data group C2.

*

*	C3 Line       :

*          NODER(1)   :  Number for first renumbered node.

*              .                  .

*          NODER(NUMR):  Number for last renumbered node.

*============================================================================

*    COULD RENUMBER NODE NUMBER 3 TO 3333

*    NODER(1)

* C3    3333

*============================================================================

*  Additional Combine Block example to illustrate simple statistics option.

*============================================================================

$COMBINE

*  ICOMB

A1   5

*  No more input required for this run.

*============================================================================

*  Additional Combine Block example to illustrate creation of ASCII file.

*============================================================================

$COMBINE

*  ICOMB

A1   4

* Title

B1 'ASCII file generation from output of earlier Combine'

B1 'ASCII file should be on DOS file: RUNCOMB.ASC'

*  NODEOT  NPOLL

B2   0      3

* Note, because NPOLL > 0 above, loads, not concentrations, will be listed

* on the interface file.

*

*    NPOS1(1) NPOS2(1) NPOS1(2) NPOS2(2)  etc.

B3     1        1         2       2       3       3

*  NUMX  NUMR

C1  0     0

*============================================================================

*               End your input data set with a $ENDPROGRAM.

$ENDPROGRAM