*

*       <<<<<<<<  SWMM 4.3 RAIN BLOCK DATA FILE >>>>>>>>>

*

*  Note, this file last updated by WCH, 4/27/94.

*

*	This is an input data file to the SWMM 4.3 (beta) Rain Block for

*       analyzing and creating a precipitation time series interface

*       file.  All lines with an asterisk in column 1 are comment lines

*       and are ignored by the program.

*

*       Input data are free format and may be up to 230 columns wide.

*       You must have a value for every data column even if the program

*       will not actually use a given value.  Alternatively, a slash, /, may

*       be used at the end of a line to indicate that remaining input

*       values should be filled with zeroes.  There must be at least one

*       space or comma between every input value.  Alphanumeric data

*       should be enclosed in single quotes.

*

*       Caution!  Data lines that are "wrapped around" (continued on

*       two or more lines) should have a blank in column 1, unless a

*       card identifier is needed.

*

*       SWMM uses both U.S. customary units and metric units.  The

*       examples use feet, cfs, acres, inches and inches/hour.  If metric

*       is specified, substitute meters, cms, hectares, millimeters and

*       millimeters/hour.

*============================================================================

*	The SW line sets up the interface files to be used or created.

*	There is one output file (#9 in this example) that will contain

*       the hourly, 15 min. or other time interval precipitation data for

*       subsequent blocks.

*============================================================================

*  NBLOCK  JIN(1)  JOUT(1)

SW    1      8       9

*============================================================================

*   Note, the rainfall interface file is created on unit JOUT.  If this

*   file already exists and is referenced through an @ line, then rainfall

*   input during this Rain Block run will be combined with rainfall already

*   stored on the referenced file.  In this way, data from multiple gages

*   with different IDs may be stored on the same rainfall interface file.

*============================================================================

*   The MM line opens the scratch files to be used by different subroutines.

*   A certain number (up to 4 for RAIN) is required for each block.

*============================================================================

*  NITCH  NSCRAT(1) NSCRAT(2) NSCRAT(3) NSCRAT(4) NSCRAT(5) NSCRAT(6) NSCRAT(7)

MM   7       1         2          3        10         11        12       13

*============================================================================

*     The @ command is used to permanently save an interface or

*     scratch file.  This line should be placed before the first SWMM

*     block call.  The format of the @ command is as follows:

*============================================================================

*Column 1    Unit number of the       Name of the interface

*            interface file saved     file (any valid DOS filename)

*            or utilized

*

@             8                       'GEO6667.DOT'

*** @             9                       'FLORN.INT'

*============================================================================

$RAIN     	Call the RAIN Block with a '$' in first column.

*============================================================================

*	Create title lines for the simulation.  There are two title lines

*       for the Rain Block.  Titles are enclosed in single quotes.

*============================================================================

*	A1 Line       :

*          SITE       : Two 80-column descriptive titles.

*============================================================================

A1  ' EXAMPLE RUN OF THE RAIN BLOCK'

A1  ' APPALACHICOLA, FLORIDA RAINFALL'

*============================================================================

*                      Optional B0 line.  May be omitted.

*

*      B0 Line        :

*

*          KODEA      : Relative to treatment of accumulated totals (totals

*                       over several hourly or 15-min intervals, instead of

*                       the actual values at each interval) in NCDC

*                       precipitation records, indicated by a special

*                       code "A":

*                        = 0, Do not include accumulated totals on

*                             interface file time series or in hourly

*                             and event summary (default, as in past).

*

*                             Options 1 and 2 available only for

*                             IFORM = 0,1,4,6 or >= 9.

*

*                        = 1, Average accumulated total over preceding

*                             intervals since last special code or beginning

*                             of day (a uniformly dispersed positioning).

*                        = 2, Insert accumulated total at its indicated

*                             time (a lumped positioning).

*          KODEPR     : For printing of presence of special codes and/or

*                       missing data in event summary table (IDECID >= 1).

*                        = 0, If present, indicate only for dates shown in

*                             in event summary table (default).

*                        = 1, If present, indicate for all dates.  This

*                             option may lengthen event summary table.

*============================================================================

*  KODEA  KODEPR

B0   1       1

*============================================================================

*	Line B1 defines the control information for the Rain Block.

*============================================================================

*	B1 Line       :

*

*       NOTE: Both hourly and 15-min. precipitation data may be analyzed

*       using IFORM = 0,1,4,6,9,10,11,12.

*       When 15-min. data are analyzed, NSCRAT(4) is required.

*       Note, that when 15-min. data are processed, annual, storm event

*       and summary statistics are all based on hourly totals, not on the

*       individual 15-min. values.  However, the 15-min. values are

*       properly stored on the interface file at 15-min. increments.

*       For IFORM = 0,1,4,6, the time interval (THISTO) is detected

*       from the NWS file.  It is not necessary to specify hourly or

*       15-min.

*

*       Only hourly data may be analyzed for IFORM = 2,5,7,8.

*

*       For IFORM = 3 (user defined) the time interval is arbitrary, but

*       1 hour or 15 min is recommended.  If the precipitation interval

*       (THISTO) is greater than 1 hour, then annual summary statistics

*       may be incorrect.  If THISTO < 1 hr, it should be an integer

*       divisor of 1 hr for proper annual and summary statistics.

*

*       Note, NOAA-NCDC precipitation data are given in terms of the

*       ending time of a precipitation value, whereas SWMM needs the

*       beginning time of the precipitation increment.  Thus, times listed

*       in the print-outs and on the interface file will be one hyetograph

*       time interval (e.g., usually 1 hr or 15 min) earlier than the times

*       listed in the NOAA-NCDC files.  I.e., SWMM hours are 0-23 (0000-

*       2345 for 15-min. data), whereas NOAA-NCDC hours are 1-24

*       (0015-2400 for 15-min. data).

*

*       Note: see examples of data for IFORM = 0,1,2,3,4,5,9,10 at end

*       of this file.

*

*          IFORM      :  Format of precipitation (rainfall) data.

*                        = 0, Post-1980 NWS format (NWS Tape Deck 3240

*                             or 3260), fixed length records (one

*                             precipitation entry per line).

*                        = 1, Post-1980 NWS format (NWS Tape Deck 3240

*                             or 3260), variable length records (all

*                             non-zero precipitation values on one line

*                             for each day).  This is also the AS ON TAPE

*                             FORMAT of the NWS and the "NCDC format" of

*                             EarthInfo.

*                        = 2, NWS Card Deck 488.

*                        = 3, User-defined rainfall format.

*                        = 4, NWS diskette rainfall format ("Release B,

*                             Condensed").  (Field definitions are similar

*                             to TD 3260 but placed on separate lines and

*                             alphanumeric values enclosed in double quotes.)

*                             Provide first 5-yr (or total) diskette (DOS)

*                             file on unit JIN and identify other input files

*                             (if any) using an @-line(s) in the input data

*                             file, i.e., immediately after the B1 line,

*                             place an @-line that identifies a

*                             new NWS DOS file (unit number and file name).

*                             (See example after B1 line.)

*                             The data on this file must follow

*                             chronologically the data already input (and

*                             include preliminary station number info etc.).

*                             If no additional data are made available,

*                             a message is printed.

*                             Because NWS input data must be converted to

*                             proper ASCII format before being read by SWMM,

*                             the converted data may be saved for future

*                             input (IFORM=6) by saving file NSCRAT(3)

*                             using an appropriate @-line.

*                        = 5, Canadian AES precipitation data format.

*                        = 6, Processed NWS diskette rainfall format

*                             ("Release B, Condensed").  Provide first

*                             diskette (DOS) file on unit JIN and

*                             identify other input files (if any) as

*                             indicated for IFORM = 4.

*                             Quotation marks in file for input using

*                             IFORM = 4 option have been changed to

*                             apostrophes in this file from a prior run

*                             using IFORM = 4 and saving

*                             resulting NSCRAT(3) file (then use saved file

*                             for unit JIN for IFORM = 6).

*                        = 7, EarthInfo NCDC rainfall display format

*                             (data retrieved from screen display, unlikely

*                             to be encountered).

*                        = 8, EarthInfo ASCII rainfall display format

*                             (data retrieved from screen display, unlikely

*                             to be encountered).

*

*                        Caution! The EarthInfo ASCII output format changed

*                        during 1993. Only use IFORM = 9 or 10 if the

*                        format of your data is the same as shown at the

*                        end of this RAIN.DOC file.  Otherwise, export

*                        EarthInfo data in the NCDC (CR/LF) format and use

*                        IFORM = 1.

*

*                        = 9, EarthInfo ASCII output format (1993), 15-min.

*                             precipitation data -- unprocessed EarthInfo

*                             data file; suggest saving processed file

*                             on NSCRAT(3).  "Processed" means placing

*                             alphanumeric data in single quotes for

*                             list-directed input.

*                        = 10 EarthInfo ASCII output format (1993), hourly

*                             precipitation data -- unprocessed EarthInfo

*                             data file; suggest saving processed file

*                             on NSCRAT(3).

*                        = 11 EarthInfo processed 15-min. ASCII precipitation

*                             data.  Input on unit JIN the file previously

*                             processed and saved on NSCRAT(3) from run

*                             using IFORM = 9.

*                        = 12 EarthInfo processed hourly ASCII precipitation

*                             data.  Input on unit JIN the file previously

*                             processed and saved on NSCRAT(3) from run

*                             using IFORM = 10.

*          ISTA       :  NWS station number, or alternatively for user

*                        defined data (IFORM = 3), an arbitrary

*                        integer station number.

*                        Note, for IFORM = 0,1,2, ISTA is a 6-digit integer,

*                        i.e., 2-digit state code plus 4-digit cooperative

*                        network index.  For IFORM = 9-12, ISTA is the 3

*                        or 4-digit cooperative network index.

*                        For IFORM = 5, ISTA is an 8-digit Canadian ID.

*                        For IFORM = 0,1,2,5,9-12, ISTA must match station

*                        number in data file.  Likewise for IFORM = 3 if

*                        a station number is read (F1 > 0).

*                        For IFORM = 4 or 6, ISTA is read from data (entry

*                        on line B1 is ignored).  For IFORM = 7 or 8, ISTA

*                        is set on B1 line and not compared to data file.

*          IDECID     :  Controls the program purpose and output.

*                        = 0, Create a rainfall interface file on JOUT.

*                        = 1, Create a rainfall interface file on JOUT, and

*                             perform the synoptic rainfall analysis.

*                        = 2, Only do the synoptic rainfall analysis.

*

*     Note, synoptic rainfall analysis is the same as performed by the

*     EPA SYNOP model.

*

*          IYBEG      :  Starting date, six digits, yr/mo/dy.

*                        If zero the program searches for beginning year.

*          IYEND      :  Ending date, six digits, yr/mo/dy.

*                        If zero the program reads all the station data.

*          IYEAR      :  Echo print of the rainfall values read

*                        = 0, Do not print all rainfall data.

*                        = 1, Print all the (non-zero) data (1-3 months/page).

*          ISUM       :  Indicator for storm event summary printout.

*                        = 0, Do not print storm event summary.

*                        = 1, Do print the storm event summary (1-2

*                             pages per year).

*============================================================================

*          Note: The following fields are not required if IDECID is 0.

*============================================================================

*          MIT        :  Minimum interevent time to separate storm

*                        events, number of hours (>_1).  Only used if storm

*                        event statistics are printed.  (See text.)

*          NPTS       :  NPTS highest events are printed in rainfall

*                        analysis tables.  If zero, all events are printed.

*          IFILE      :  Indicator for storm event formatted file creation.

*                        = 0, Do not save storm event data on a file.

*                        = 1, Save the storm event data on a formatted

*                        NSCRAT(1).  User must insure that this file will

*                        be permanently saved by appropriate JCL or by

*                        using the @ function as described above.

*          A          :  Plotting position parameter (see text for

*                        explanation).  0.4 recommended, or else use zero

*                        for Weibull plotting position.

*          NOSTAT     :  Indicator for storm event statistics printout:

*

*      NOSTAT controls the printing of the event tables for volume, average

*  intensity, storm duration, and interevent duration.  In all cases,

*  No = 0, Yes = 1.  The control information is entered as a four digit

*  integer number with each column controlling a different table.

*          First column   - table of storm volume by return period.

*          Second column  - table of average intensity by return period.

*          Third column   - table of storm duration by return period.

*          Fourth column  - table of interevent duration by return period.

*      For example, enter 1111 to print all four tables, 1000 to print the

*   first table only, 1100 to print the first two tables, and 0000

*   or 0 to bypass all table printing.  In all cases only the top NPTS

*   events are printed.

*============================================================================

*  POST 1980 NWS RAINFALL FORMAT

*  IFORM ISTA  IDECID  IYBEG  IYEND IYEAR ISUM MIT NPTS IFILE A NOSTAT

B1   0  080211  1     660000 670000   1     1  12  50     0  0.4  1111

*============================================================================

*   Below is an example of a line that could be used to provide file

*   information to continue input for IFORM = 4 or 6.  The @ symbol should

*   be in the first column.  File NEWDATA.PRE would contain a

*   continuation of Release B Condensed precipitation data,

*   including all the normal header information.  These data would follow

*   chronologically from prior data read in for this station.  The unit

*   number, 4, is arbitrary but should not duplicate other unit numbers

*   listed on the SW or MM lines.

*

*  @   4  'C:\SWMMDATA\NEWDATA.PRE'

*============================================================================

*	Line B2 defines the control information for a user-defined

*               rainfall time series.  Required only if IFORM = 3

*               on data group B1.

*============================================================================

*  A maximum of 5000 precipitation values per year may be input in file JIN

*  (no limit on number of years), one value per line, each value with a date

*  and time (and optional station ID).  Note, 24 precipitation values per

*  line may be entered according to special criteria through the use of

*  optional line B3, explained below.

*

*	B2 Line       :

*          THISTO     :  Rainfall time interval, minutes.

*          METRIC     :  Metric input/output.

*                        = 0, Use U.S. customary units.

*                        = 1, Use metric units, denoted by brackets [].

*          KUNIT      :  Standard rainfall units.

*                        = 0, intensity, in./hr [mm/hr], or

*                        = 1, volume, in. [mm].

*                        (Note: other units, e.g., hundredths of an inch,

*                        may be used by using CONV below.)

*          FIRMAT     :  Rainfall format (character data; enclose in single

*                        quotes).

*                        Should include integer fields for station number,

*                        year, month, day, hour, minute and an F, E or G

*                        field for rainfall.  Rainfall may not be read as

*                        integer (I-format) unless using B3 line -- see

*                        below.

*          CONV       :  Multiply by CONV to convert non-standard rainfall

*                        units to in./hr [mm/hr] (KUNIT = 0) or in. [mm]

*                        (KUNIT = 1).  (Conversion should be to mm

*                        if METRIC = 1.)

*

*                        NOTE: The value for the hour of precipitation should

*                        vary from zero (midnight) to 23 (11:00 p.m. = 2300).

*                        E.g., hour = 2 and minute = 15 gives 0215 for the

*                        precipitation starting hour.  This is the

*                        default procedure.  If desired, the hour can vary

*                        from 1 to 24 (midnight).  E.g., hour = 2 and minute

*                        = 15 gives 0115 for the precipitation starting

*                        hour.  To choose this option (hour varying from

*                        1 to 24), enter CONV as a negative number

*                        (e.g., CONV = -1.0).  CONV will be reset in

*                        the program to its correct positive value.

*

*          F1         :  Field position for station number.

*                        If F1 is zero a station number will not be read.

*          F2         :  Field position for year.  Required.

*          F3         :  Field position for month.  Required.

*          F4         :  Field position for day.  Required.

*          F5         :  Field position for hour.  Required.

*          F6         :  Field position for minute.  Required.

*          F7         :  Field position for precipitation.  Required.

*                        Rainfall must be either first (1) or last (6 or 7)

*                        field.  SWMM considers positive numbers to be

*                        rainfall and negative numbers to be snowfall.

*============================================================================

*     Use line B2 if IFORM = 3 on data line B1.

*     USER DEFINED INPUT RAINFALL TIME SERIES ==> "USRN.DAT"

*   THISTO METRIC KUNIT   FIRMAT       CONV  F1 F2 F3 F4 F5 F6 F7

* B2  15.0    0     0   '(I5,5I3,F4.0)'  1.0  1  2  3  4  5  6  7

*============================================================================

*      Optional data group for entry of 24 precipitation values per line.

*   This line may be omitted.  If used, this line should follow the B2 line.

*

*       B3              Group identifier

*

*       NUVAL           = 24 = number of precipitation values entered on

*                       each B2 line for variable F7.  If this option is

*                       used, the year, month and day are entered (in the

*                       sequence defined by F2, F3 and F4 on line B2 and

*                       in the fields specified by FIRMAT on line B2),

*                       followed by 24 (hourly) precipitation values in

*                       100th's of an inch (METRIC=0) or mm x 100 (METRIC=1).

*                       For example, an intensity of 0.2 in./hr should be

*                       entered as 20 (METRIC=0), and an intensity of 30

*                       mm/hr should be entered as 3000 (METRIC=1).

*                       *** Rainfall is read in integer format.***

*                       The integer format (e.g., 24I5) is included

*                       in parameter FIRMAT on line B2.  There is no

*                       multiplication by parameter CONV of these

*                       data entries.

*

*                       Note that by manipulation of the format specified

*                       in parameter FIRMAT, the data can be arranged

*                       in groups of 12 values on two lines.  For instance,

*                       if year, month and day occupy the first three fields

*                       of each line as real values, followed by 12

*                       precipitation values as integers, the following

*                       FIRMAT is an example: '(3I4, 12I5,/,12X,12I5)'

*                       Of course, a field position may be left for

*                       a station number if need be.

*============================================================================

*  This is a one-day example using format listed above.  Note that zeroes

*  are not necessary to fill-in empty fields because a Format statement

*  is used, but zeroes are included below for clarity.

* YEAR MON DAY   RAIN VALUES....

*  86   2   5   20    0    0    0    0    0   10   40   110   10    0   0

*  86   2   5    0   10    0    0    0    0    0    0     0    0    0   0

*

*                            End of Rain Block Data

*               End your input data set with a $ENDPROGRAM.

*               or a $BLOCKNAME for another SWMM block.

$ENDPROGRAM

*

*  Brief examples of data formats.  Remember that asterisk in column 1 is

*  necessary to maintain comment lines in this RAIN.DOC file.

*

*  IFORM = 0, Appalachacola, Florida, April 27-28, 1964

*

*HPD08021100HPCPHI19640400270011900 00002  &&&

*HPD08021100HPCPHI19640400270012000 00004  &&&

*HPD08021100HPCPHI19640400270012100 00003  &&&

*HPD08021100HPCPHI19640400270012200 00001  &&&

*HPD08021100HPCPHI19640400270012300 00012  &&&

*HPD08021100HPCPHI19640400280010100 00005  &&&

*HPD08021100HPCPHI19640400280010300 00001  &&&

*

* IFORM = 1, Indianapolis, Indiana, January 1-14, 1987

*

*HPD12425900HPCPHI19870100010020100000000  2500000000

*HPD12425900HPCPHI19870100020050300000001  0600000001  0700000002  0800000002  2500000006

*HPD12425900HPCPHI19870100090131300000003  1400000009  1500000004  1600000008  1700000010  1800000006  1900000004  2000000007  2100000003  2200000004  2300000001  2400000002  2500000061

*HPD12425900HPCPHI19870100100031900000001  2400000001  2500000002

*HPD12425900HPCPHI19870100110020100000001  2500000001

*HPD12425900HPCPHI19870100140021300000003  2500000003

*

* IFORM = 2, Minneapolis, Minnesota, January 1-4, 1960

*

*2154356001011-  -  -  -  -  -  -  -  -  -  -  -

*21543560010120020020030030020030020050030020030010031                         02

*2154356001021002001001002004002002002001-  -  -

*2154356001022-  -  -  -  -  -  -  -  -  -  -  -  0017                         04

*2154356001041-  -  -  -  -  -  -  -  -  -  -  -

*2154356001042-  -  -  001-  -  -  -  -  -  -  -  0001                         05

*

* IFORM = 3, Hypothetical user-defined.  This is USRN.DAT refered to in text.

*

*14922 80 02 01 00 00 .1

*14922 80 02 01 00 30 .1

*14922 80 02 01 14 00 .1

*14922 80 02 01 15 00 .1

*14922 80 02 01 18 00 .1

*14922 80 02 01 19 00 .1

*14922 80 02 23 12 30 .1

*14922 80 02 23 12 45 .1

*14922 80 02 23 15 00 .1

*14922 80 02 23 18 00 .1

*14922 80 02 23 19 00 .1

*14922 80 02 24 17 00 .1

*14922 80 02 24 18 00 .1

*14922 80 02 25 12 00 .1

*14922 80 02 26 12 00 .1

*14922 80 02 27 12 00 .1

*14922 80 02 27 15 00 .1

*14922 80 02 27 15 30 .1

*14922 80 03 04 12 00 .05

*14922 80 03 04 12 30 .05

*14922 80 03 04 14 00 .05

*14922 80 03 05 00 00 .05

*14922 80 03 05 00 30 .05

*

* IFORM = 4, Greensboro-High Point, North Carolina, July 1-12, 1948

*

*1,'HPD'

*2,'GREENSBORO,HIGH P, NC'

*3,313630,313630,0,0,0,0,0,0,0,0

*4,1948,07,1952,12

*5,'HPCP',' ',' ',' ',' '

*8,948,7,1,'HI',2

*9,1,0,'0','0'

*9,25,0,'0','0'

*8,948,7,11,'HI',2

*9,3,1,'0','0'

*9,25,1,'0','0'

*8,948,7,12,'HI',3

*9,13,1,'0','0'

*9,14,4,'0','0'

*9,25,5,'0','0'

*

* IFORM = 5, Unidentified Canadian station 61060009, November 13-18, 1978

* Note, rainfall in mm.

*

*61060009781113123000000000000000000000000000000000000000000000000000000000000000000000000000200150007002400370031

*61060009781114123002200170006000300000000000000000000000000000000000000000000000000000000000000000000000000000000

*61060009781115123000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000

*61060009781116123000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000

*61060009781117123000000000000000000000000000000000000000000000000000000130019002800220013000900260024004600460002

*61060009781118123000200040005000000000000000000000000000000000000000000000000000000000000000000000000000000000000

*

* IFORM = 9, Yaquina Bay, Oregon, November 1-7, 1980

*

*OR,9581,1,in,11/01/1980,0.00,,0.10,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,0.10,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,0.10,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,0.10,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,0.10,,0.20,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,0.70,

*OR,9581,1,in,11/02/1980,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,0.10,,.,,.,,.,,.,,.,,.,,.,,.,,.,,0.10,

*OR,9581,1,in,11/03/1980,.,,.,,.,,.,,.,,.,,.,,.,,0.10,,.,,.,,.,,.,,.,,.,,0.10,,.,,0.10,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,0.30,

*OR,9581,1,in,11/05/1980,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,0.10,,.,,.,,.,,0.10,,.,,.,,.,,0.10,,0.10,,.,,0.10,,0.10,,.,,0.10,,.,,.,,0.10,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,0.80,

*OR,9581,1,in,11/06/1980,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,0.10,,.,,.,,.,,.,,0.10,,.,,0.10,,.,,0.10,,.,,0.10,,.,,0.10,,.,,0.10,,.,,.,,.,,0.10,,.,,.,,.,,.,,.,,0.10,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,0.20,,.,,.,,.,,.,,0.10,,0.20,,.,,0.10,,1.50,

*OR,9581,1,in,11/07/1980,0.10,,0.10,,.,,.,,0.10,,.,,.,,.,,.,,.,,.,,.,,.,,0.10,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,0.10,,0.40,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,0.10,,.,,.,,.,,.,,0.10,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,1.10,

*

* IFORM = 10, Eugene, Oregon, January 1-9, 1970

*

*OR,2709,2,in,01/01/1970,0.00,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,0.00,,

*OR,2709,2,in,01/03/1970,.,,.,,.,,.,,.,,.,,.,,.,,.,,0.02,,.,,.,,0.02,,0.01,,0.03,,.,,.,,.,,0.08,,.,,0.01,,.,,.,,.,,0.17,,

*OR,2709,2,in,01/07/1970,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,0.01,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,.,,0.01,,

*OR,2709,2,in,01/08/1970,.,,.,,.,,.,,.,,.,,.,,0.01,,.,,0.02,,.,,.,,.,,0.01,,.,,0.02,,.,,.,,.,,0.04,,0.04,,0.13,,0.08,,0.04,,0.39,,

*OR,2709,2,in,01/09/1970,0.01,,0.06,,0.07,,0.18,,0.16,,0.09,,0.08,,0.08,,0.03,,0.06,,0.04,,0.02,,0.03,,0.06,,0.06,,0.07,,0.06,,0.11,,0.11,,0.04,,0.02,,0.02,,.,,.,,1.46,,